Supporting Information for:

Episodic river flow transports tens of billions of microplastics annually to Monterey Bay, California

Matthew S. Savoca, Pam Krone, Rebecca Roberts, Angela Anguiano, Hazel Vaquero, Scott Coffin, Win Cowger

Supplemental Methods

## Polymer density assignment and buoyancy analysis

Cumulative particle-size-distribution (C-PSD) fitting and size rescaling

Sensitivity of extrapolation to single- versus piecewise power-law structure

## Comparison with published PSDs

Supplemental Figures

Supplemental Tables

References

## Supplemental Methods

## Polymer density assignment and buoyancy analysis

Particles retained following µFTIR identification and quality screening were classified by morphology as fibers or fragments, with fibers defined by an aspect ratio ≥3 by ref. 1. Spectral-library material classifications were mapped to 13 polymer-density groups: polyethylene, polypropylene, styrenic polymers, polytetrafluoroethylene (PTFE), polyester/polyethylene terephthalate (PET), polycarbonate, polyurethane, polyamide/acrylamide, acrylic polymers, vinyl polymers, halogenated vinyl polymers, silicone polymers, and cellulose derivatives. Polymer densities were assigned using literature values from ref. 2. Material classifications that could not be assigned to one of these polymer groups were assigned a density of 1.10 g cm-³, corresponding to the average density reported for microplastics in surface waters from ref. 3.

Particles were classified as buoyant, neutrally buoyant, or settling relative to freshwater density (1.00 g cm⁻³). In the present dataset, only polyethylene and polypropylene were classified as buoyant under freshwater conditions. To test for density-driven vertical sorting within rivers, particles were grouped as buoyant or non-buoyant and the proportion of buoyant particles was compared between surface and subsurface samples. The surface-versus-subsurface comparison was evaluated using a one-sided two-sample test of proportions, with the alternative hypothesis that the proportion of buoyant particles was greater in surface than subsurface samples. When expected cell frequencies were <5, a one-sided Fisher's exact test was used instead.

Cumulative particle-size-distribution fitting and size rescaling

Particle-size distributions (PSDs) were characterized from measured particle lengths using the cumulative particle-size-distribution (C-PSD) framework4. Environmental microplastic PSDs are commonly approximated by the differential power-law relationship:

NL= bLa

Where N(L) represents particle abundance as a function of particle length L, a is the differential power-law exponent, and b is the scaling coefficient. In log-log space, this relationship is linear with slope a.  Rather than fitting conventional bin-normalized PSDs (BN-PSDs), we fitted cumulative particle abundances following ref. 4. Conventional BN-PSD regression can produce slope estimates that depend systematically on the selected bin width, whereas cumulative fitting substantially reduces this binning bias and produces estimates comparable to maximum-likelihood fits of raw particle-size observations. For the cumulative distribution, the number of particles greater than or equal to a given particle length follows:

N≥L= b'La'

Where a′ and b′ are the fitted cumulative slope and scaling coefficient, respectively. The cumulative and differential slopes are related by:

a' = a + 1

and therefore, the differential PSD slope reported in this study was calculated as:

a = a' - 1

The differential scaling coefficient can similarly be recovered from the cumulative parameters as described by ref. 4.

The lower and upper particle-size limits over which each C-PSD was fitted were determined using the two-step limit-of-detection (LOD) procedure4. This procedure was used because environmental PSDs may deviate from the expected power law at both ends of the measured size distribution. At the lower end, incomplete recovery or analytical detection can result in undercounting of small particles, whereas at the upper end, increasingly rare large particles may be insufficiently represented at the sampled volume. Both processes can bias the estimated power-law slope if affected size classes are included in the regression.

For each analysis group, particle lengths were binned and both the BN-PSD and C-PSD were calculated. A preliminary ordinary least-squares (OLS) regression was fitted across the full C-PSD. Bins with positive residuals from this preliminary fit were retained as candidates for the final regression window, while bins below the peak of the BN-PSD were excluded to reduce the influence of small-particle undercounting. Bins above 5000 µm were also excluded because they fall outside the conventional microplastic size range.

All possible contiguous candidate windows containing at least three size bins were subsequently evaluated using OLS regression of log10 cumulative particle abundance against log10 particle length. Candidate windows with R2 < 0.90 were excluded. Among the remaining windows, the longest qualifying window was selected for the final C-PSD fit, with ties resolved using R2. If no candidate window met these criteria, the widest available candidate range was retained as a fallback. C-PSDs were fitted separately by particle morphology and for each of the four rivers (Carmel, Pajaro, Salinas, and San Lorenzo). The resulting cumulative slopes were converted to the differential-slope convention described above before subsequent comparisons and size rescaling.

To determine whether independently fitted river-level PSD slopes differed by more than expected from sampling uncertainty, we used Cochran’s Q heterogeneity test. Each river-specific differential slope was weighted by the inverse of its sampling variance, 1SEi2, where SEi is the standard error of the fitted slope. Cochran’s Q was then calculated and evaluated against a χ² distribution with k - 1 degrees of freedom, where k was the number of valid river-specific slope estimates. We additionally calculated I2 to quantify the percentage of observed variation among slopes attributable to heterogeneity rather than sampling error. Together, these statistics were used to evaluate whether independently fitted river PSDs differed systematically among locations.

Fitted power laws were used to rescale measured particle concentrations between specified particle-size ranges following established microplastic size-alignment approaches4,5. For a differential PSD,

dNdL= kLa

where is L particle length, a is the differential PSD slope, and k is the scaling coefficient. The predicted number of particles within a size interval [L1, L2] is obtained by integrating the differential PSD:

NL1-L2= L1L2kLadL= ka+1L2a+1- L1a+1

To rescale a measured concentration to a target size range, we calculated a correction factor (CF) as the ratio of predicted particle abundance over the target size interval to that over the measured size interval:

CF =NLtarget,min,Ltarget,maxNLmeasured,min,Lmeasured,max

The measured concentration was multiplied by CF to obtain the corresponding concentration over the target size interval. Because the same fitted PSD is integrated in the numerator and denominator, the scaling coefficient k cancels, and the correction factor depends only on the fitted differential slope and specified size limits. Power-law extrapolation assumes that the fitted relationship extends beyond the empirically supported fitting window. In addition, the upper tail of a C-PSD can deviate from the fitted relationship when rare large particles are insufficiently sampled4. We therefore restricted slope estimation to the empirically supported C-PSD fitting window and evaluated sensitivity to the single-power-law assumption separately.

Extrapolation of measured concentrations

The microplastic concentration associated with the operational 50-500 µm sampling fraction was rescaled to the full 1-5000 µm size range. The 50-500 µm interval represents the nominal size fraction defined by the stacked sampling sieves rather than a strict bound on measured particle length. Specifically, the correction factor (CF) was calculated as the ratio of predicted particle abundance across the target 1-5000 µm range to predicted abundance across the empirically measured 50-500 µm range:

CF50-500 → 1-5000=N1-5000N50-500

and the extrapolated concentration as:

C1-5000= C50-500* CF50-500 → 1-5000

Because the same fitted PSD is used in the numerator and denominator, the scaling coefficient k cancels and the correction factor depends only on the fitted differential slope and the specified size limits. For the pooled river-water C-PSD, the fitted cumulative slope was -1.920 ± 0.035 SE and was converted to the corresponding differential slope, a = -2.92, following a = a′ -1. The approximate 95% confidence limits of the differential slope were calculated as a ± 1.96 SE, and each slope limit was propagated independently through the integration to obtain a slope-derived 95% confidence interval for the extrapolated concentration. Because increasingly negative slopes predict progressively greater numbers of small particles, the steeper confidence limit defines the upper concentration bound and the shallower limit defines the lower bound. These bounds quantify uncertainty in the estimated slope but do not incorporate structural uncertainty associated with extrapolating a single power law beyond the empirically supported fitting window; model-form uncertainty was evaluated separately using the piecewise sensitivity analysis described below.

## Sensitivity of extrapolation to single- versus piecewise power-law structure

The primary PSD extrapolation assumes that a single power law fitted within the empirically supported C-PSD window extends toward smaller, unmeasured particle sizes. However, observed PSDs can also reflect size-dependent environmental transport. Differences in particle settling and rise velocities can produce different apparent power-law relationships among particle-size classes, even when the underlying particle population follows a single power law6. Because the transition between transport regimes depends on particle properties and local flow conditions rather than a universal particle-size threshold, extrapolation across substantially different size classes may be sensitive to the assumption of a single PSD exponent. We therefore evaluated a two-segment power-law model as a structural sensitivity analysis.

The piecewise model was fitted to the same C-PSD window selected for the primary single-slope model; the LOD/window-selection procedure was not repeated. Candidate breakpoints were evaluated at bin edges within this window, with at least three bins required on each side. For each candidate breakpoint, an ordinary least-squares model was fitted to log10 cumulative particle abundance as a function of log10 particle length, segment (fine versus coarse), and their interaction, allowing independent slopes on either side of the breakpoint. The breakpoint yielding the minimum Akaike Information Criterion (AIC)7 was retained.

Support for the piecewise relative to the single-slope model was evaluated as:

ΔAIC = AICpiecewise- AICsingle

Negative ΔAIC values therefore indicate greater support for the piecewise model, whereas positive values favor the single-slope model. Because both models were fitted over the same C-PSD window, this comparison isolates the effect of allowing the PSD slope to change without introducing differences in fitting-window selection.

For concentration rescaling under the piecewise model, the fine- and coarse-size power laws were integrated separately using the power-law integration described above. The coarse-segment scaling coefficient was adjusted to maintain continuity of the differential PSD at the fitted breakpoint. Size intervals spanning the breakpoint were divided into fine and coarse portions, integrated separately, and summed. The resulting piecewise correction factor was compared with the primary single-slope correction factor to quantify the sensitivity of extrapolated concentrations to PSD model form.

The piecewise model produced a substantially smaller correction factor than the single-slope model because the fitted fine-particle slope was shallower than the coarse-particle slope. For river water, the correction factor decreased from 22,465 under the single-slope model to 2,403 under the piecewise model, a 9.3-fold reduction (piecewise/single-slope ratio = 0.107). These values demonstrate that extrapolated abundance is highly sensitive to the assumed PSD structure; however, the direction of this sensitivity should not be interpreted as universal. Independent field observations have demonstrated that single-power-law extrapolation can also underestimate smaller particles. In the St. Louis Estuary, for example, single-power-law extrapolation predicted 1.3–12 particles L⁻¹ over 5–5000 µm, whereas flow cytometry measured 600–1600 particles L⁻¹ in the same system8. Similarly, size-aligned 1–5000 µm concentrations were, on average, approximately 600-fold greater than concentrations estimated by single-slope extrapolation4. Thus, the piecewise analysis is interpreted as a test of model-form sensitivity rather than evidence that the piecewise estimate necessarily represents a lower or more accurate concentration.

## Comparison with published PSDs

We benchmarked fitted PSD slopes from Monterey Bay rivers against selected published microplastic PSD estimates to place the observed distributions in a broader environmental context. This comparison was targeted rather than intended as a systematic review and was designed to span complementary approaches to microplastic sampling and PSD characterization. Sources included particle-resolved µFTIR measurements of Dutch river suspended solids9, environmental C-PSD reference values and the MPSizeBase compilation4, high-resolution particle-resolved observations reported by ref.3, and a mesh-based literature synthesis from ref.10.

Published slopes were taken as reported rather than independently re-estimated from underlying particle observations. Because studies expressed PSD slopes using different conventions, values were converted where necessary to the common signed differential-slope convention (a) used in this study. Positive power-law exponents were converted to the corresponding negative differential PSD exponent as appropriate.

For the expanded comparison shown in Fig. SX, individual ocean PSD slopes were obtained from the MPSizeBase supplementary dataset4. These values represent PSDs from the studies compiled in MPSizeBase and were not independently re-derived from the primary datasets. That compilation included 81 published PSDs and reported mean differential slopes of -1.86 ± 0.36 for fibers and -2.66 ± 0.68 for fragments. These morphology-specific differences were considered when comparing Monterey Bay river PSDs with published environmental distributions.

Supplementary Figures

Fig. SX. Visal of FTIR analytical workflow

Figure SX. C-PSD power law fits for particle lengths for different matrices (colored points/lines) for single-slope (left panel) vs two-segment fits (right panel). Particle length (x-axis, um) and relative abundance (y-axis) are both log-transformed. Vertical dashed lines indicate LOD bounds for each of the fit regression lines.

Figure SX. Buoyant-polymer (PP + PE) fraction by matrix and river depth stratum, with binomial 95% Cis.

Figure SX. C-PSD power-law fits by matrix/location (color) x particle shape (y-axis facet) x particle attribute (x-axis facet) with fitted LOD windows (darker points and regression lines). Relative bin abundance is on the y-axis, with size on x-axis (units depend on each particle attribute). Both axes are log10 scale.

Figure SX. C-PSD power law fits by particle attribute (x-axis facet) by river (x-axis labels) with points representing median fit values and error bars corresponding to 95% CI’s. Only power law fits for all shapes combined are shown.

Figure SX. Particle-length PSD slopes (a), this study vs. published literature. Fits by shape (fragment/fiber/all) and by individual river sites are labels, with point estimates and corresponding 95% CIs. While refs.3,9,10 used different power law fitting algorithms to this study, ref.4 and the MPSizeBase data were all derived using same method in this study.

Supplemental Tables

| Polymer type | Total particles | Percent of plastic particles |
| --- | --- | --- |
| poly(propylene) | 1,298 | 34.4% |
| poly(acrylamide/amid)s | 1,103 | 29.2% |
| poly(ethylene) | 456 | 12.1% |
| polystyrenes (polyphenylethylenes, -methylstyrene) | 334 | 8.8% |
| poly(esters/ethers/diglycidylethers/terephthalates)s | 176 | 4.7% |
| polyacrylates (propenoates) | 130 | 3.4% |
| polyurethanes (isocyanates) | 67 | 1.8% |
| polyhaloolefins (vinylhalides) | 60 | 1.6% |
| polymethacrylates | 31 | 0.8% |
| polysiloxanes | 26 | 0.7% |
| poly(tetrafluoroethylene) | 25 | 0.7% |
| polyvinylesters | 24 | 0.6% |
| cellulose derivatives (ether cellulose) | 16 | 0.4% |
| ethylene-vinyl acetate | 6 | 0.2% |
| polyhydroxy(meth)acrylates | 6 | 0.2% |
| polyvinylalcohols | 6 | 0.2% |
| polyimides | 4 | 0.1% |
| acrylonitrile butadiene styrene (abs) | 3 | 0.1% |
| polyacrylonitriles (nitriles) | 2 | 0.1% |
| poly(ether)ketones (polyketones) | 1 | 0.0% |
| polycarbonates | 1 | 0.0% |
| polyethersulfones (polysulfone) | 1 | 0.0% |
| polyvinylethers | 1 | 0.0% |

Table S1. Polymer types from all river samples. Count of all 23 polymer types of particles 50-500 m as identified by FTIR.

Table SX. Lab blank levels from Moore Institute (smaller size class)

| group | mippr_id | count |
| --- | --- | --- |
| MBMS_01 | MIPPR_MBMS_PrcdrlBlnk_7Feb24 | 12 |
| MBMS_02 | MIPPR_PB_23JUL24 | 1 |
| MBMS_03 | MIPPR_PB_08JAN25 | 2 |
| MBMS_06A | MIPPR_PB_30JAN25 | 0 |
| MBMS_06B | MIPPR_PB_7FEB25 | 9 |
| MBMS_06C | MIPPR_PB_13MAR25 | 10 |
| MBMS_07A | MIPPR_PB_05AUG25 | 2 |
| MBMS_07B | MIPPR_PB_15AUG25 | 4 |
| MBMS_07C | MIPPR_PB_27AUG25 | 2 |
| MBMS_11A | MIPPR_PB_24APR26 | 0 |
| MBMS_11B | MIPPR_PB_29APR26 | 8 |
| MBMS_11C | MIPPR_PB_06MAY26 | 19 |

Table SX. Lab blanks from Hopkins Marine Station (larger size class): 58 lab blanks (one for every day of labwork), median of two fibers per blank.

| Date | Total particles | Fibers | Fragments | Films | Nurdles | Foam |
| --- | --- | --- | --- | --- | --- | --- |
| 8/12/2024 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8/16/2024 | 3 | 3 | 0 | 0 | 0 | 0 |
| 8/25/2024 | 1 | 1 | 0 | 0 | 0 | 0 |
| 4/26/2024 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9/2/2024 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6/28/2024 | 4 | 4 | 0 | 0 | 0 | 0 |
| 9/5/2024 | 2 | 2 | 0 | 0 | 0 | 0 |
| 9/26/2024 | 2 | 2 | 0 | 0 | 0 | 0 |
| 9/28/2024 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4/5/2024 | 1 | 1 | 0 | 0 | 0 | 0 |
| 4/10/2024 | 2 | 2 | 0 | 0 | 0 | 0 |
| 11/17/2024 | 0 | 0 | 0 | 0 | 0 | 0 |
| 11/21/2024 | 1 | 1 | 0 | 0 | 0 | 0 |
| 12/12/2024 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1/14/2025 | 3 | 3 | 0 | 0 | 0 | 0 |
| 1/27/2025 | 3 | 3 | 0 | 0 | 0 | 0 |
| 2/17/2025 | 2 | 2 | 0 | 0 | 0 | 0 |
| 2/24/2025 | 3 | 2 | 0 | 1 | 0 | 0 |
| 2/27/2025 | 3 | 3 | 0 | 0 | 0 | 0 |
| 3/3/2025 | 5 | 3 | 2 | 0 | 0 | 0 |
| 3/6/2025 | 6 | 4 | 2 | 0 | 0 | 0 |
| 3/10/2025 | 5 | 4 | 1 | 0 | 0 | 0 |
| 3/17/2025 | 5 | 4 | 1 | 0 | 0 | 0 |
| 3/20/2025 | 3 | 3 | 0 | 0 | 0 | 0 |
| 3/24/2025 | 3 | 3 | 0 | 0 | 0 | 0 |
| 3/27/2025 | 2 | 2 | 0 | 0 | 0 | 0 |
| 3/31/2025 | 2 | 2 | 0 | 0 | 0 | 0 |
| 4/7/2025 | 3 | 3 | 0 | 0 | 0 | 0 |
| 4/10/2025 | 2 | 2 | 0 | 0 | 0 | 0 |
| 4/17/2025 | 3 | 3 | 0 | 0 | 0 | 0 |
| 4/21/2025 | 2 | 2 | 0 | 0 | 0 | 0 |
| 4/24/2025 | 3 | 3 | 0 | 0 | 0 | 0 |
| 4/28/2025 | 1 | 1 | 0 | 0 | 0 | 0 |
| 5/5/2025 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5/8/2025 | 2 | 2 | 0 | 0 | 0 | 0 |
| 5/13/2025 | 1 | 1 | 0 | 0 | 0 | 0 |
| 5/14/2025 | 2 | 2 | 0 | 0 | 0 | 0 |
| 5/19/2025 | 1 | 1 | 0 | 0 | 0 | 0 |
| 5/20/2025 | 9 | 9 | 0 | 0 | 0 | 0 |
| 5/27/2025 | 6 | 6 | 0 | 0 | 0 | 0 |
| 6/10/2025 | 5 | 5 | 0 | 0 | 0 | 0 |
| 6/11/2025 | 2 | 2 | 0 | 0 | 0 | 0 |
| 6/17/2025 | 1 | 1 | 0 | 0 | 0 | 0 |
| 6/18/2025 | 4 | 3 | 1 | 0 | 0 | 0 |
| 6/19/2025 | 1 | 1 | 0 | 0 | 0 | 0 |
| 7/1/2025 | 9 | 6 | 3 | 0 | 0 | 0 |
| 7/3/2025 | 2 | 2 | 0 | 0 | 0 | 0 |
| 7/8/2025 | 5 | 4 | 1 | 0 | 0 | 0 |
| 7/10/2025 | 9 | 5 | 4 | 0 | 0 | 0 |
| 7/15/2025 | 2 | 2 | 0 | 0 | 0 | 0 |
| 7/17/2025 | 6 | 6 | 0 | 0 | 0 | 0 |
| 7/22/2025 | 2 | 2 | 0 | 0 | 0 | 0 |
| 7/24/2025 | 1 | 1 | 0 | 0 | 0 | 0 |

References

(1)	Cole, M. A Novel Method for Preparing Microplastic Fibers. Sci Rep 2016, 6 (1), 34519. https://doi.org/10.1038/srep34519.

(2)	Hidalgo-Ruz, V.; Gutow, L.; Thompson, R. C.; Thiel, M. Microplastics in the Marine Environment: A Review of the Methods Used for Identification and Quantification. Environ. Sci. Technol. 2012, 46 (6), 3060–3075. https://doi.org/10.1021/es2031505.

(3)	Kooi, M.; Primpke, S.; Mintenig, S. M.; Lorenz, C.; Gerdts, G.; Koelmans, A. A. Characterizing the Multidimensionality of Microplastics across Environmental Compartments. Water Research 2021, 202 (July). https://doi.org/10.1016/j.watres.2021.117429.

(4)	Segur, T.; Hough, I.; Dobiasova, N.; Voisin, D.; Richon, C.; Angot, H.; Thomas, J. L.; Sonke, J. E. Using the Power Law Size Distribution to Extrapolate and Compare Microplastic Number and Mass Concentrations in Environmental Media. Micropl.&Nanopl. 2026, 6 (1), 45. https://doi.org/10.1186/s43591-026-00205-5.

(5)	Koelmans, A. A.; Mohamed Nor, N. H.; Hermsen, E.; Kooi, M.; Mintenig, S. M.; De France, J. Microplastics in Freshwaters and Drinking Water: Critical Review and Assessment of Data Quality. Water Research 2019, 155, 410–422. https://doi.org/10.1016/j.watres.2019.02.054.

(6)	Hoffman, M. J.; DiBenedetto, M. H. How Size-Dependent Settling Can Both Bias and Inform Microplastics Observations. Environ. Res. Commun. 2026, 8 (6), 065061. https://doi.org/10.1088/2515-7620/ae76df.

(7)	Symonds, M. R. E.; Moussalli, A. A Brief Guide to Model Selection, Multimodel Inference and Model Averaging in Behavioural Ecology Using Akaike’s Information Criterion. Behavioral Ecology and Sociobiology 2011, 65 (1), 13–21. https://doi.org/10.1007/s00265-010-1037-6.

(8)	Thomas, A.; Marchand, J.; Schwoerer, G. D.; Minor, E. C.; Maurer-Jones, M. A. Size Distributions of Microplastics in the St Louis Estuary and Western Lake Superior. Environ. Sci. Technol. 2024, 58 (19), 8480–8489. https://doi.org/10.1021/acs.est.3c10776.

(9)	Wang, S.; Mintenig, S. M.; Wu, J.; Koelmans, A. A. Implications of Method- and Instrument-Based Size Detection Limits in μFTIR-Based Microplastic Analysis. Talanta 2026, 296, 128417. https://doi.org/10.1016/j.talanta.2025.128417.

(10)	Zhao, B.; Richardson, R. E.; Huang, Y.; You, F. Physically-Based Mesh Selectivity Correction Model for Standardized Microplastic Abundance Estimates in Aquatic Environment. Water Research 2026, 299, 125833. https://doi.org/10.1016/j.watres.2026.125833.