Episodic river flow drives microplastic transport to Monterey Bay, California

Matthew S. Savoca1,2, Pam Krone1, Rebecca Roberts1, Angela Anguiano3, Hazel Vaquero3, Scott Coffin4, Win Cowger3

Affiliations

1California Marine Sanctuary Foundation, Monterey, CA, USA

2Hopkins Marine Station, Stanford University, Pacific Grove, CA, USA

3Moore Institute for Plastic Pollution Research, Long Beach, CA, USA
4California Office of Environmental Health Hazard Assessment, Sacramento, CA, USA

Abstract

Intermittent rivers may deliver microplastics to coastal oceans in concentrated pulses, yet their contribution to land-sea plastic transport remains poorly resolved, particularly for smaller particles. From 2023 to 2025, we sampled the four major rivers that empty into Monterey Bay – a large open-ocean bay on the U.S. west coast – collecting samples before, during, and after periods of heavy winter and spring rainfall. In a total of 11,400 L of river water, we identified 3,777 microplastics in the 50-500 µm fraction using FTIR, while an additional 237 putative microplastics were visually identified in the 500-5000 µm size fraction. Average concentration in the 50-500 µm fraction was 0.33 microplastics L⁻¹ and varied substantially through time but did not differ significantly among rivers, sampling seasons, or depths. While FTIR analysis confirmed 23 different polymer types, over 84% were polypropylene (34.3%), polyamides (29.2%), polyethylene (12.1%), and polystyrene (8.8%). The first major storm of the 2023-2024 wet season transported an estimated ~10 billion 50-500 µm microplastics from the four rivers to coastal waters (25th–75th percentile concentration scenarios: 6.2–32.1 billion). Microplastic abundance increased steeply with decreasing size, and power-law size-distribution modeling yielded a model-based  concentration of approximately 600 particles L⁻¹ (slope-derived 95% CI: 460-790 particles L⁻¹) across the full 1-5000 µm microplastic size range. Together, these results identify episodic discharge and analytical size range as major controls on estimates of riverine microplastic delivery and suggest that conventional measurements capture only a small fraction of microplastics that may affect downstream coastal ecosystems.

Key words: Plastic pollution, microplastic, microfiber, flood, hydrology, land-to-sea transport, particle size distribution

Synopsis:

Storm-driven river discharge is a key control on microplastic flux to coastal waters, while abundant unresolved smaller particles may substantially increase the magnitude and biological impacts of these inputs.

Introduction

Plastic pollution is a defining environmental challenge of the Anthropocene, with microplastics (< 5 mm) now detected in every major habitat on Earth, from alpine snowfields to deep-sea sediments1. Despite this, the dominant pathways and processes governing the transport of plastics from land to the ocean remain poorly understood. Most plastics are neutrally or positively buoyant, enabling prolonged suspension in aquatic environments, exacerbating risks to aquatic wildlife and increasing the likelihood to be carried to distant waters and disparate ecosystems2. Rivers are a key pathway by which plastic pollution of all sizes reach the ocean3–6; however, microplastic concentrations – which ultimately influence their biological effects – in rivers vary by several orders of magnitude; some of this variability is inherent and some is the result of sample collection, processing, and analysis7. Resolving microplastic concentrations in rivers and their timing and flux to the sea is vital to understanding the threat of this contaminant to coastal communities and ecosystems.

Global estimates of plastic transport from land to the ocean have changed considerably over the past decade. Early global models suggested that rivers transfer approximately 2-4 Mt of plastic to the ocean annually3,4.  These estimates are now thought to be biased high due to limited observations and simplifying assumptions. More recent efforts that incorporate improved waste management data, hydrology, and river networks estimate global riverine plastic emissions of approximately 0.5 Mt per year5. In parallel, the spatial attribution of riverine plastic emissions has shifted from a paradigm dominated by a small number of large rivers to one in which plastic export is far more diffuse. Initially, only two dozen major rivers were considered dominant contributors3, but updated analyses have found that more than 1,000 rivers worldwide may each contribute meaningfully to global plastic emissions8.

While initial estimates tended to focus on macroplastics3,9,10, the past five years, have seen rapid growth in studies quantifying microplastic concentrations and fluxes in rivers. These studies reveal that river microplastic transport is strongly mediated by hydrology, varying dramatically over time in response to discharge, seasonality, and storm events11–14. However, the strongest determinant of microplastic concentrations is often human population density15–17. Importantly, river systems differ widely in flow regimes: some rivers are perennial, others ephemeral, and many – particularly in Mediterranean-climate regions – are intermittent, flowing seasonally or episodically in response to heavy precipitation. These hydrologic differences influence when, how, and in what quantities microplastics are mobilized and exported to downstream environments11,14,18,19.

An additional source of variability in reported microplastic concentrations is the particle-size range captured by different sampling and analytical approaches. Environmental microplastic abundance generally increases nonlinearly with decreasing particle size, such that measurements using coarse meshes can exclude a numerically dominant fraction of smaller particles20–25. Particle-size distributions (PSDs) can be described using power-law relationships, providing a framework for aligning measurements made across different size ranges and estimating abundance below directly observed particles. However, such extrapolations depend strongly on the fitted PSD and require explicit consideration of uncertainty in extending the relationships of observed microplastics toward smaller particles25. Resolving both hydrologic variability and particle-size distributions is therefore necessary to constrain riverine microplastic abundance and transport.

The four major rivers discharging into Monterey Bay, and the adjacent Carmel Bay, are the San Lorenzo, Pajaro, Salinas, and Carmel Rivers from north to south (Fig. 1A). Together, these rivers drain watersheds that span a wide range of land uses and population densities, including urban development in the San Lorenzo River watershed (Santa Cruz, California), intensive agricultural landscapes in the Pajaro and Salinas River watersheds, and a smaller, affluent suburban catchment associated with the Carmel River. Despite these contrasting watershed characteristics and their direct connection to a biologically productive coastal ocean, plastic pollution in these rivers has not been previously quantified. All four systems share a strongly seasonal, intermittent hydrology, with minimal or no flow for much of the year punctuated by episodic winter and spring storm events that dominate annual discharge. The prolonged low-flow conditions may promote the accumulation of plastics within river channels, sediment, and stagnant sections. Subsequent high-discharge events may rapidly mobilize microplastics that have accumulated within river channels, sediments, and low-flow reaches to the coastal ocean. Similar pulse-driven microplastic transport has been documented in other river systems11,13,14,18,19.

Plastic pollution in the four rivers in our study have not previously been measured, and the magnitude, timing, and polymer composition of riverine microplastic inputs to the region is unknown despite their recurrent connection to a highly productive coastal ecosystem. In contrast, microplastics have been repeatedly documented throughout Monterey Bay, in seawater from the surface to depths of 1,000 m26,27, across trophic levels from the bathy- to epipelagic26,28,29, and in commercially harvested species29. Together, these river-open bay systems form a coupled terrestrial-marine observatory that enables microplastics to be used as tracers of watershed processes, hydrologic variability, and the timing and magnitude of downstream exposure and risk to coastal ecosystems.

Assessing potential exposure of aquatic organisms to riverine microplastics requires resolving not only when particles are transported, but also their concentrations, size distributions, polymer composition, and behavior under realistic hydrologic conditions. Yet few studies have repeatedly quantified particle-resolved microplastic concentrations across multiple size classes in intermittent rivers. By leveraging the intermittent hydrology and contrasting watershed characteristics of rivers discharging into Monterey Bay, we quantify microplastic concentrations, particle-size and shape distributions, polymer composition, and flux as a function of river discharge. We additionally evaluate whether polymer density is associated with vertical particle sorting between surface and subsurface waters. Finally, we use empirically fitted power-law particle-size distributions to place our directly measured concentrations within the full 1-5000 µm microplastic size range and evaluate the sensitivity of these extrapolations to alternative particle-size-distribution structures. Together, these analyses link hydrologic transport with particle-scale properties to constrain both the timing and magnitude of microplastic delivery from intermittent rivers to the coastal ocean.

Materials and Methods

Field sites

River samples were collected at four major Central Coast systems – the Salinas, Pajaro, San Lorenzo, and Carmel Rivers (Fig. 1) – representing mixed agricultural and urbanized watersheds. Specifically, the Salinas and Pajaro sites are classified as agricultural watersheds and the Carmel and San Lorenzo sites are considered urban. All sampling sites were bridges over the river, within 3 km of where the river discharged into Monterey Bay (Fig. 1). This was done to sample the thalweg of the river to avoid bank effects and obtain samples that were most representative of what was being transported downriver. In the first season, we sampled each river’s stagnant lower reaches before the first storm events. Additional sampling was conducted as near to a storm event as possible to capture near peak flow, except for the final monitoring events in 2025 (Fig. 2). Because the end of the wet season was approaching, sampling needed to occur prior to the subsidence of river discharge. All sampling took place from December to May, which is the season when the region receives > 90% of its annual precipitation.

Microplastic collection methods

At each site, surface and subsurface water samples were obtained using a Teledyne ISCO peristaltic pump connected to two, stacked stainless-steel sieves (500 µm and 54 µm mesh). Each ISCO unit pumped approximately 200 L of water per sample at 4 L min⁻¹ over a 50-minute period. Field blanks (one per river per year) were collected simultaneously by exposing a secondary sieve stack to ambient air and rinsing with deionized water to capture potential airborne contamination. Samples were preserved with 10 mL of 70% isopropyl alcohol and stored in airtight glass containers for up to a year until processing.

### Laboratory Methods

For the larger size class of microplastics in our study (500-5000 µm), samples underwent visual isolation under a Nikon SMZ745 stereomicroscope (3-300x). Suspected plastics were transferred to glass slides with double sided sticky tape and classified by morphology as in ref.30. Color, morphology, and approximate size (within 100 µm) were recorded for each putative microplastic.

The Moore Institute for Plastic Pollution Research is a California Environmental Laboratory Accreditation Program (ELAP)-accredited lab for microplastic analysis in drinking water (SWB-MP1-Rev1) and conducted the analytical chemistry and filtration analysis for this study as dictated by their standard approach but added new methods for prior extraction due to the abundance of particles in river water compared to drinking water. Samples underwent size separation and were sieved using 20 μm sieves. For microplastics (> 20 µm), samples were digested using 20% KOH and density separated using filtered (< 1 μm glass fiber filter) seawater collected at Belmont Pier, Long Beach, CA. The decanted density fluid was filtered onto one or more 25 mm stainless steel filters with a 5 μm pore size for FTIR analysis. Filters were processed on a Thermo Scientific Nicolet™ iN™10 MX FTIR microscope in reflectance mode with an 8X2 MCT array detector (4000-675 cm⁻¹, 16 cm⁻¹ resolution, 25 um step size).

As laboratory procedures were refined, the amount of sample analyzed by FTIR was standardized. Initially, the entire sample was analyzed, which led to multiday runs per sample and was not feasible within the study's budget and likely unnecessary given only 100 microplastic particles are needed for standard statistical confidence of < 10% error31 and each filter analyzed 1000s of particles. Later, analysis was limited to a maximum of three aliquots (subsamples) per sample, and the aliquot-to-sample volume ratio was recorded to extrapolate particle counts to the entire sample. Quality control included procedural blanks, field blanks, and duplicate subsampling to quantify contamination and analytical uncertainty. Positive controls and recovery rates were assessed using standardized lab spikes. One spike, one procedural blank, and one air blank were run per 20 samples. Spikes consisted of polyethylene, polyolefins, cellulose acetate, and PVC dust reference material. Blank filter paper was used for air blanks to track contamination in the laboratory whenever samples from this project were being processed and analyzed. Fibers were visually identified and hand-picked under a microscope. Glass microscope slides were prepared with two strips of double-sided tape, leaving an opening in the center. Individual fibers were placed between the tape strips, counted, classified by color, measured, and photographed.

Particle density and buoyancy estimation

Particles retained from µFTIR imaging were classified by shape as fibers or fragments, with fibers defined by an aspect ratio ≥ 3 from ref.32). Polymer-specific densities were assigned to spectroscopically identified microplastics using published polymer densities 33. Material classes that could not be assigned a polymer-specific density were assigned a default density of 1.10 g cm⁻³, corresponding to the average value reported for microplastics in surface waters34. Particles were then classified as buoyant, neutrally buoyant, or settling relative to freshwater (1.00 g cm⁻³); only polypropylene and polyethylene were classified as buoyant in this dataset. We compared the proportion of buoyant particles between surface and subsurface river samples to evaluate evidence for density-driven vertical sorting. Polymer-density assignments and statistical procedures are described in the Supporting Information.

Microplastic flux estimation

River flow data during the period of our study was downloaded from freely-available USGS river flow gauges (San Lorenzo River site ID: 11161000, Pajaro River site ID: 11159500; Salinas River site ID: 11152500, Carmel River, site ID: 11143250). River flow rate was converted from ft3 s-1 to m3 s-1 for analysis and visualization. We defined our first major storm period if the hourly flow for that river exceeded the 90th percentile of the average annual flow for > 12 consecutive hours. USGS discharge data were retrieved using the dataRetrieval package35 in R.

We used the median, 25th-percentile, and 75th-percentile microplastic concentrations from the first sampling season (2023-2024) for each river to represent central, lower, and upper concentration scenarios, respectively. River discharge measurements were aggregated to hourly means. Hourly microplastic flux was estimated as discharge (m³ s⁻¹) × microplastic concentration (particles L⁻¹) × 1,000 L m⁻³ × 3,600 s h⁻¹. Hourly estimates were cumulatively summed over the first major storm period to estimate total microplastic transport under each concentration scenario. River discharge was plotted concurrently as a scaled time series to illustrate the relationship between discharge and cumulative microplastic transport.

Particle size distribution (PSD) and power-law fitting

Microplastic PSDs were characterized using a power-law framework because environmental microplastic abundance generally increases exponentially with decreasing particle size24,25,34  Length-based cumulative particle-size distributions (C-PSDs) were constructed following Segur et al. (2026) by fitting the cumulative number of particles greater than or equal to a given particle length [N(≥L)] as a function of particle length (L) in log-log space using binned count data. This approach reduces the bin-width bias associated with conventional bin-normalized PSD fitting. Lower and upper size limits for fitting were determined using the C-PSD limit-of-detection procedure of ref.25, which identifies the approximately linear portion of each distribution while excluding size ranges affected by incomplete detection of small particles or insufficient sampling of rare large particles.

Power-law slopes were estimated separately by particle morphology and for each of the four rivers to characterize variation in particle-size distributions. Fitted power laws were integrated across specified particle-size intervals to rescale measured concentrations to common size ranges, allowing observations from the two measured size fractions to be evaluated within a common PSD framework24,25. Because size-dependent transport can produce different apparent power-law relationships among particle-size classes, extrapolation beyond the observed size range may be sensitive to the assumption that a single power law adequately represents the underlying PSD36. We therefore additionally evaluated a two-segment power-law model as a sensitivity analysis.

To estimate microplastic concentrations across the full 1-5000 µm size range, the fitted differential power law was integrated over both the target (1-5000 µm) and empirically measured (50-500 µm) size ranges. A size-range correction factor was calculated as the ratio of predicted particle abundance in the target range to that in the measured range and multiplied by the empirical 50-500 µm concentration. Uncertainty in the extrapolated concentration was estimated by propagating the 95% confidence interval of the fitted differential PSD slope through the same integration. The 165-385 µm limit-of-detection (LOD) window was used to estimate the power-law slope and does not represent the empirical concentration interval; the latter was defined operationally by the 50- and 500-µm sampling sieves. Additional equations and sensitivity analyses addressing uncertainty in the assumed power-law structure are provided in the Supporting Information.

Data Processing and Analysis

Spectroscopic identification of polymer type was conducted using Open Specy 37, an open-source software designed for automated processing and classification of microplastic spectra. All collected FTIR spectra maps were batch analyzed through an automated pipeline for analyzing hyperspectral images with R 4.4.2 and the OpenSpecy v1.5.3 package (Figure SX). Raw spectral maps were pre-processed using range restriction (800–2200 cm⁻¹ and 2420–3200 cm⁻¹), absolute third-order Gaussian smoothing, and baseline correction of first-order derivative transformation. Spectral matching was performed against Open Specy’s reference library of more than 17,000 FTIR spectra, with identification thresholds set to correlation coefficients ≥ 0.6 and a signal-to-noise threshold of 0.01. Final particle identifications were overlaid onto the original hyperspectral map to produce a spatial representation of particle locations and material classifications.

Microplastic concentrations and particle characteristics were summarized using medians, median absolute deviations (MAD), and 25th and 75th percentiles to reduce sensitivity to skewed distributions and extreme values. Particles were categorized by morphology, color, and polymer type. Polymer composition was compared between river-water samples, field blanks, and laboratory blanks using sample-level polymer proportions. For each sample, counts of each polymer class were divided by the total number of identified plastic particles to calculate the proportional polymer composition, with undetected polymer classes assigned a proportion of zero. Mean polymer proportions and 95% confidence intervals were estimated separately for each sample type by bootstrapping whole samples with replacement (10,000 iterations), thereby retaining the compositional structure of polymers within individual samples. Differences in overall polymer composition among sample types were initially evaluated using PERMANOVA based on Bray-Curtis dissimilarities; because multivariate dispersion differed significantly among sample types, polymer-specific comparisons were used to evaluate differences between environmental samples and blanks. For each polymer, differences in mean proportional abundance between river-water samples and field or laboratory blanks were estimated by bootstrap resampling of samples, with significance assessed from the bootstrap distribution of between-group differences and P-values adjusted for multiple comparisons using the Benjamini-Hochberg procedure38 . Differences in small microplastic concentrations (particles L⁻¹) among rivers were evaluated using a linear model. Because concentrations were right-skewed, they were log10-transformed prior to analysis. River, sampling season, and sample depth (surface or subsurface) were included as fixed effects. Overall effects of each predictor were evaluated using Type II analysis of variance. Model assumptions of normality and homogeneity of variance were assessed by visual inspection of normal Q-Q and residual-versus-fitted plots, respectively.

Differences in particle-size distributions among rivers were evaluated using the independently fitted river-level C-PSD slopes described above. Heterogeneity among river-specific slopes was tested using Cochran’s Q, with slopes weighted by their inverse variance (See Supporting Information). The statistic was evaluated against a χ² distribution with degrees of freedom, where was the number of river-specific slope estimates, and was used to quantify the proportion of observed variation attributable to heterogeneity among rivers rather than sampling error. Additional details of C-PSD fitting and slope comparisons are provided in the Supporting Information. All data processing, statistical analyses, and visualization were conducted in R v4.5.2, using the tidyverse39 packages dplyr, tidyr, ggplot2, and purrr. For multivariate analyses we used the vegan package40 in R, and the car package41 for Type II analysis of variance.

Results and Discussion

Empirical microplastic concentration in river water

Across 57 river-water samples (29 surface samples, 28 subsurface samples) representing 11,400 L of filtered water, the overall concentration of spectroscopically identified small microplastics (50–500 µm) was 0.33 particles L⁻¹, based on 3,777 identified particles. Larger suspected microplastics (500–5000 µm) that were extracted manually (n = 237 particles) were approximately 20-fold less abundant (Table 1). Concentrations also varied substantially through time: 14% of samples contained > 1 microplastic L⁻¹, with some of the highest concentrations observed under low-flow conditions prior to the onset of winter rainfall (Fig. 2). By the end of the rainy season, concentrations were as much as two orders of magnitude lower (Figs. 2 and 5).

Overall, river, sampling season, and sampling depth explained relatively little variation in small microplastic concentrations, and the full model was not statistically significant (F₅,₅₁ = 1.01, P = 0.421, R² = 0.09). Small microplastic concentrations did not differ significantly among rivers after accounting for sampling season and depth (Type II ANOVA: F₃,₅₁ = 1.10, P = 0.356). Relative to the San Lorenzo River, concentrations were lower in the Pajaro River (β = −0.21, t = −0.92, P = 0.361), Salinas River (β = -0.30, t = -1.32, P = 0.194), and Carmel River (β = -0.40, t = -1.75, P = 0.087), although none of these differences were statistically significant. Sampling season also had no significant overall effect (F₁,₅₁ = 1.14, P = 0.291), with concentrations tending to be higher during Season 2 relative to Season 1 (β = 0.18, t = 1.07, P = 0.291). Similarly, concentrations did not differ significantly between sampling depths (F₁,₅₁ = 0.29, P = 0.593), with surface concentrations slightly lower than subsurface concentrations (β = -0.09, t = -0.54, P = 0.593).

Polymer composition and buoyancy results

Polymer composition differed between river-water samples and blanks (Fig. 4), providing evidence that the polymer types observed in rivers was not explained by field or laboratory contamination. Polypropylene was the most abundant polymer in river-water samples, comprising a mean of 29.4% (95% CI: 24.9-34.2%) of identified plastics, followed by polyamides at 22.8% (18.1-27.6%) and polyethylene at 14.2% (11.1-17.4%). Polypropylene proportions did not differ significantly between river water and either field or laboratory blanks after correction for multiple comparisons. In contrast, polyamides were substantially more abundant in river water than in laboratory blanks (mean difference = 20.6 percentage points, 95% CI: 15.2–26.0; BH-adjusted P < 0.001), although the difference from field blanks was not significant. Interestingly, the concentration of polyamides in our river water samples and field blanks was roughly twice the frequency expected based on their respective shares of global plastic production, roughly 15% for both42 Several less abundant polymers were also enriched in river water relative to both blank types, including polyacrylates (river-water mean = 4.1%, 95% CI: 2.6-5.8%), polysiloxanes (0.8%, 0.2-1.6%), and cellulose derivatives (0.2%, 0.0-0.4%; all BH-adjusted P < 0.01). Conversely, polyethylene and polyhaloolefins were significantly more abundant in laboratory blanks than river water (mean differences = -18.8 and -13.2 percentage points, respectively; BH-adjusted P = 0.024 and 0.041).

Polymer density did not provide evidence for strong vertical sorting within the rivers. Contrary to the expectation for local surface accumulation of low-density polymers, the fraction of particles classified as buoyant was numerically greater in subsurface samples (31.6%, 160/507) than at the surface (23.7%, 166/700), and the hypothesized surface enrichment was not supported statistically (one-sided two-proportion test, P = 0.999; Fig. SX7). Thus, particle density alone appears insufficient to explain the observed vertical distribution of riverine microplastics. This is consistent with the expectation that turbulent mixing and other hydrodynamic processes can counteract density-driven settling or rising, particularly in flowing waters.

Microplastic flux estimation for the 50-500 size fraction

The first major storm of the 2023–2024 wet season transported an estimated ~10 billion (6.19 - 32.07 billion MPs 25th-75th percentile range) microplastics in the 50-500 µm from the four rivers to the nearshore coastal ocean (Fig. 5). We expect that cumulative particle export increased most rapidly as river flow peaked, even though some of the highest measured concentrations occurred before the first major storm events. This apparent decoupling between concentration and flux is particularly important for intermittent Mediterranean-climate rivers, where long periods of low or absent discharge limit oceanward transport and may permit particles to accumulate within stagnant reaches, sediments, and their watersheds before winter rains restore hydrologic connectivity. The high pre-storm concentrations followed by lower concentrations later in the wet season are therefore consistent with a seasonal accumulation-and-flushing mechanism13,19,43, although resolving the relative contributions of channel storage, sediment remobilization, and watershed inputs will require more highly resolved storm-event sampling.

The Pajaro and Salinas Rivers contributed the greatest estimated particle export during the first major storm despite concentrations that were not significantly higher than those of the other rivers. Their greater contribution instead reflects the importance of river flow in determining total microplastic discharge. More broadly, our estimates should be considered conservative because they include only the four major rivers in this region and only the first major storm period. Additional inputs include subsequent winter storms, smaller tributaries that were not measured here, direct stormwater discharge44,45, and treated wastewater effluent46–48. these flux estimates also include only the empirically quantified 50-500 µm fraction. Because particle abundance generally increases exponentially with decreasing size21–24, restricting measurements to this size range likely excludes a numerically dominant fraction of the microplastic population. We therefore used the observed particle-size distributions to quantify how much microplastic abundance may lie outside the directly quantified size range.

Particle-size distributions reveal substantial unmeasured microplastics

As in numerous other studies, our microplastic abundance increased strongly with decreasing particle size, and pooled river-water C-PSDs were well described by power-law relationships (all particles a’: = -1.920 ± 0.035 SE, R² = 0.988). Particle morphology strongly influenced the size distribution: fragments exhibited a substantially steeper C-PSD slope (a′ = -2.739 ± 0.038, R² = 0.993) than fibers (a′ = -1.671 ± 0.046, R² = 0.970; Fig. 6). This difference is consistent with the dimensional dependence of plastic fragmentation, whereby fragmentation of three-dimensional particles produces a more rapid increase in abundance with decreasing size than fragmentation of elongated fibers25. Converted to the differential-slope convention used for comparison among studies, our fragment slope (a = -3.74) was steeper than the fiber slope (a = -2.67), consistent with morphology-dependent differences reported across environmental microplastic datasets25.

Particle-size distributions also differed strongly among rivers despite the absence of significant differences in measured microplastic concentrations. River-specific C-PSD slopes ranged from -1.493 ± 0.068 in the Salinas River and -1.503 ± 0.074 in the Carmel River to -1.975 ± 0.050 in the San Lorenzo River and -2.711 ± 0.063 in the Pajaro River (Fig. 6). Heterogeneity among river-specific slopes was substantial (Cochran’s Q = 227.4, P < 0.001, I2 = 98.7%), indicating pronounced differences among watersheds in the relative abundance of small versus large microplastics. Our differential fragment PSD slope (a = -3.74) was within the range reported from particle-resolved µFTIR measurements of Dutch river suspended solids23. Differences from slopes derived using coarser-mesh sampling further underscore the importance of analytical size range when comparing reported microplastic concentrations, because exclusion of the numerically abundant smaller size fraction can substantially alter estimated PSD slopes and concentrations22,25.

The steep river-water PSD indicates that the operationally measured 50-500 µm fraction represents only a small proportion of microplastics predicted across the full 1-5000 µm microplastic size range. Using the pooled differential PSD slope (a = -2.920 ± 0.035 SE), integration of the fitted power law yielded a correction factor of approximately 1,850 from the 50-500 µm fraction to the full 1-5000 µm range. Applying this correction to the empirically measured mean concentration of 0.33 particles L⁻¹ yielded a model-derived estimate of approximately 600 microplastics L⁻¹ (slope-derived 95% CI: 460–790 microplastics L⁻¹) across 1-5000 µm. The fitted slope was estimated over the empirically supported 165-385 µm C-PSD window and extrapolated beyond that range, particularly toward smaller particle sizes; consequently, this confidence interval reflects uncertainty in the estimated slope but not uncertainty in the assumption that a single power law extends to 1 µm. A two-segment model was strongly favored over the single-slope model (ΔAIC = -44.0), confirming that the magnitude of the extrapolation is sensitive to assumed PSD structure (Supporting Information). Thus, the estimated 1-5000 µm concentration should be interpreted as a model-based size-aligned estimate rather than a directly measured concentration, while illustrating the potentially large fraction of microplastics omitted when measurements are restricted to conventional size ranges.

Conclusions

By combining high-volume river sampling with particle-resolved µFTIR analysis, we quantified microplastic concentrations, composition, and transport across four intermittent rivers draining to Monterey Bay. Across 57 river-water samples representing approximately 11,400 L of water, we identified 3,777 microplastics in the 50-500 µm size fraction, complemented by visual characterization of 237 putative microplastics in the 500-5000 µm size range. Despite contrasting watersheds, measured microplastic concentrations differed relatively little among rivers. Instead, hydrology emerged as the dominant control on export. The first major storm alone transported an estimated ~10 billion empirically quantified 50-500 µm microplastics from these four rivers to coastal waters. This strong dependence on episodic discharge is consistent with evidence that floods can mobilize microplastics stored within river channels and sediments11,13,19,43,44 and highlights the importance of resolving short-lived high-flow events when estimating land-to-ocean plastic fluxes.

Our results also demonstrate how environmental microplastic abundance depend strongly on the lower particle-size limit of measurement. Spectroscopic analysis revealed steep size distributions in which microplastic abundance increased rapidly as measured particles became smaller. Further applying the fitted PSD to our empirical 50-500 µm concentration produced a model-based estimate of approximately 600 particles L⁻¹ across the full 1-5000 µm microplastic size range, compared with 0.33 microplastics L⁻¹ measured in our operational 50-500 µm fraction. This extrapolation remains sensitive to PSD model structure and should not be interpreted as a direct measurement; rather, it demonstrates the potentially large numerical contribution of particles below conventional analytical size limits. Improved empirical measurements below 50 µm therefore represent an important next step for both flux estimation and ecological risk assessment. Smaller particles overlap more strongly with the ingestible size range of plankton and other small aquatic organisms49, and microplastic ingestion has been associated with altered feeding, growth, development, and reproduction in experimental studies, although broader environmental consequences remain uncertain50. Nanoplastics (< 1 µm), which were outside the scope of this study and should not be extrapolated from our microplastic PSD without additional evidence, represent a further unresolved component of particle abundance and exposure. Understanding the ecological consequences of these concentrated, storm-driven inputs, particularly the unmeasured smallest particles, may therefore be essential to resolving the true scale of microplastic exposure and effects in coastal ecosystems.

Fig. 1. Field sites and sampling. A) The four major rivers flowing into Monterey Bay – the San Lorenzo, Pajaro, Salinas, and Carmel Rivers – highlighted in blue. The blue arrows represent the location of the USGS water gauges upriver of our sampling sites. The inset map shows the sampling region outlined in black in context in central California. The orange arrows represent our sampling locations at each river. B) sampling from a bridge over the Pajaro River. Inset photo shows sampling basket including the peristaltic pump and stacked sieves.

Fig. 2. Microparticle concentration and river flow. Gray vertical arrows indicate sampling events; each vertical red bar is the average value of the samples taken at the indicated date at each river. A) Concentration of small microplastics (50-500 m) in river water overlaid with river flow data. B) Concentration of larger suspected microplastics (500-5000 m) in river water overlaid with river flow data.

Fig. 3. Concentrations of particle type by river and sampling season. Concentrations of small microplastics (50-500 m) by A) the four river sampling sites and B) by the two sampling years.

Fig. 4. Polymer types of small microplastics. A) Polymer type by river shows marginal differences. B) Polymer types by blank, which found no polyamides and higher levels of polyethylene (particularly in lab blanks) as compared to what was observed in samples of river water, shown in panel A. C) bars show the median value; error bars display median absolute deviation (MAD) of small microplastics in the samples collected at the surface and subsurface of the rivers.

Fig. 5. Microplastic concentrations and estimated flux during the first major storm period. A) River discharge (blue) and measured microplastic concentrations (50-500 µm; maroon) during the 2023-2024 sampling season. Gray shading denotes the first major storm period used for flux estimation; points represent individual surface and subsurface samples. B) River discharge (blue) and cumulative estimated microplastic transport (maroon) during the first major storm period. Solid maroon lines represent estimates based on extrapolations of median river-specific concentrations from the 2023-2024 season; dashed lines represent estimates based on the corresponding 25th and 75th percentiles.

Fig. 6. C-PSD particle length fits by morphology and river. Power-law fits for river water samples by morphology and river location. Data within valid LOD windows have darker opacity than points outside the windows. Regression lines drawn only within valid LOD ranges.

Table 1. Microplastic concentrations in each river by depth. Median with 25th-75th percentile in parentheses. Microplastic concentrations of either size class did not vary significantly between rivers or sampling depth.

| River | Sample depth | MPs per L (50–500 µm) | Sample size | MPs per L (500–5000 µm) | Sample size | small:large MP ratio |
| --- | --- | --- | --- | --- | --- | --- |
| San Lorenzo | surface | 0.400 (0.282–0.758) | 7 | 0.005 (0.005–0.009) | 6 | 80 |
|  | subsurface | 0.200 (0.113–1.305) | 7 | 0.005 (0.005–0.018) | 7 | 40 |
| Pajaro | surface | 0.220 (0.083–0.415) | 7 | 0.020 (0.016–0.024) | 6 | 11 |
|  | subsurface | 0.180 (0.077–0.823) | 7 | 0.025 (0.020–0.028) | 7 | 7 |
| Salinas | surface | 0.106 (0.068–0.344) | 8 | 0.015 (0.014–0.025) | 8 | 7 |
|  | subsurface | 0.235 (0.108–0.428) | 7 | 0.025 (0.005–0.035) | 8 | 9 |
| Carmel | surface | 0.050 (0.050–0.305) | 7 | 0.010 (0.005–0.018) | 7 | 5 |
|  | subsurface | 0.165 (0.103–0.253) | 7 | 0.015 (0.010–0.033) | 7 | 11 |

References

(1)	Thompson, R. C.; Courtene-Jones, W.; Boucher, J.; Pahl, S.; Raubenheimer, K.; Koelmans, A. A. Twenty Years of Microplastic Pollution Research—What Have We Learned? Science 2024, 386 (6720), eadl2746. https://doi.org/10.1126/science.adl2746.

(2)	Murphy-Hagan, C.; Gray, A. B.; Singh, S.; Hapich, H.; Cowger, W.; Seeley, M. E.; Waldschläger, K. Microplastic Characterization and Transport Mode — A Flow-Integrated Approach to Sampling Urban Waterways. Environmental Research 2025, 269, 120908. https://doi.org/10.1016/j.envres.2025.120908.

(3)	Lebreton, L. C. M.; Van Der Zwet, J.; Damsteeg, J. W.; Slat, B.; Andrady, A.; Reisser, J. River Plastic Emissions to the World’s Oceans. Nature Communications 2017, 8, 15611. https://doi.org/10.1038/ncomms15611.

(4)	Schmidt, C.; Krauth, T.; Wagner, S. Export of Plastic Debris by Rivers into the Sea. Environ. Sci. Technol. 2017, 51 (21), 12246–12253. https://doi.org/10.1021/acs.est.7b02368.

(5)	Strokal, M.; Vriend, P.; Bak, M. P.; Kroeze, C.; van Wijnen, J.; van Emmerik, T. River Export of Macro- and Microplastics to Seas by Sources Worldwide. Nat Commun 2023, 14 (1), 4842. https://doi.org/10.1038/s41467-023-40501-9.

(6)	van Emmerik, T. H. M.; Frings, R. M.; Schreyers, L. J.; Hauk, R.; de Lange, S. I.; Mellink, Y. A. M. River Plastic Transport and Deposition Amplified by Extreme Flood. Nat Water 2023, 1 (6), 514–522. https://doi.org/10.1038/s44221-023-00092-7.

(7)	Weiss, L.; Ludwig, W.; Heussner, S.; Canals, M.; Ghiglione, J.-F.; Estournel, C.; Constant, M.; Kerhervé, P. The Missing Ocean Plastic Sink: Gone with the Rivers. Science 2021, 373, 107–111.

(8)	Meijer, L. J. J.; van Emmerik, T.; van der Ent, R.; Schmidt, C.; Lebreton, L. More than 1000 Rivers Account for 80% of Global Riverine Plastic Emissions into the Ocean. Science Advances 2021, 7 (18), eaaz5803. https://doi.org/10.1126/sciadv.aaz5803.

(9)	Cowger, W.; Gray, A.; Brownlee, S.; Hapich, H.; Deshpande, A.; Waldschläger, K. Estimating Floating Macroplastic Flux in the Santa Ana River, California. Journal of Hydrology: Regional Studies 2022, 44, 101264. https://doi.org/10.1016/j.ejrh.2022.101264.

(10)	van Emmerik, T.; Kieu-Le, T.-C.; Loozen, M.; van Oeveren, K.; Strady, E.; Bui, X.-T.; Egger, M.; Gasperi, J.; Lebreton, L.; Nguyen, P.-D.; Schwarz, A.; Slat, B.; Tassin, B. A Methodology to Characterize Riverine Macroplastic Emission Into the Ocean. Front. Mar. Sci. 2018, 5, 00372. https://doi.org/10.3389/fmars.2018.00372.

(11)	Eppehimer, D. E.; Hamdhani, H.; Hollien, K. D.; Nemec, Z. C.; Lee, L. N.; Quanrud, D. M.; Bogan, M. T. Impacts of Baseflow and Flooding on Microplastic Pollution in an Effluent-Dependent Arid Land River in the USA. Environ Sci Pollut Res 2021, 28 (33), 45375–45389. https://doi.org/10.1007/s11356-021-13724-w.

(12)	Lahon, J.; Handique, S. Impact of Flooding on Microplastic Abundance and Distribution in Freshwater Environment: A Review. Environ Sci Pollut Res 2023, 30 (56), 118175–118191. https://doi.org/10.1007/s11356-023-30819-8.

(13)	Tanaka, M.; Nihei, Y. How Flooding Rivers Deliver Plastic to the Ocean: A Case Study of Microplastic and Mesoplastic Load–Discharge Relationships. Water Research 2026, 291, 125175. https://doi.org/10.1016/j.watres.2025.125175.

(14)	Treilles, R.; Gasperi, J.; Tramoy, R.; Dris, R.; Gallard, A.; Partibane, C.; Tassin, B. Microplastic and Microfiber Fluxes in the Seine River: Flood Events versus Dry Periods. Science of The Total Environment 2022, 805, 150123. https://doi.org/10.1016/j.scitotenv.2021.150123.

(15)	Correa-Araneda, F.; Pérez, J.; Tonin, A. M.; Esse, C.; Boyero, L.; Díaz, M. E.; Figueroa, R.; Santander-Massa, R.; Cornejo, A.; Link, O.; Jorquera, E.; Urbina, M. A. Microplastic Concentration, Distribution and Dynamics along One of the Largest Mediterranean-Climate Rivers: A Whole Watershed Approach. Environmental Research 2022, 209, 112808. https://doi.org/10.1016/j.envres.2022.112808.

(16)	Kunz, A.; Schneider, F.; Anthony, N.; Lin, H.-T. Microplastics in Rivers along an Urban-Rural Gradient in an Urban Agglomeration: Correlation with Land Use, Potential Sources and Pathways. Environmental Pollution 2023, 321, 121096. https://doi.org/10.1016/j.envpol.2023.121096.

(17)	Shi, H.; Yu, D.; Yin, L.; Sui, Y.; Liu, Y.; Qiao, S.; Wang, W.; Zheng, W.; Ding, D. Source-Sink Process of Microplastics in Watershed-Estuary-Offshore System. Journal of Cleaner Production 2022, 338, 130612. https://doi.org/10.1016/j.jclepro.2022.130612.

(18)	Felton, A.; Farner, S.; Day, L.; Gibbs-Huerta, S. E.; Zamarripa, B.; Hutchinson, J.; Felton, A.; Farner, S.; Day, L.; Gibbs-Huerta, S. E.; Zamarripa, B.; Hutchinson, J. Spatial–Temporal Characterization of Microplastics in the Surface Water of an Urban Ephemeral River. Microplastics 2025, 4 (1). https://doi.org/10.3390/microplastics4010009.

(19)	Hurley, R.; Woodward, J.; Rothwell, J. J. Microplastic Contamination of River Beds Significantly Reduced by Catchment-Wide Flooding. Nature Geoscience 2018, 11 (4), 251–257. https://doi.org/10.1038/s41561-018-0080-1.

(20)	Kang, J.; Youn, O.; Lee, K.; Kyoung, Y.; Joon, W. Marine Neustonic Microplastics around the Southeastern Coast of Korea. Marine Pollution Bulletin 2015, 96, 304–312. https://doi.org/10.1016/j.marpolbul.2015.04.054.

(21)	Lindeque, P. K.; Cole, M.; Coppock, R. L.; Lewis, C. N.; Miller, R. Z.; Watts, A. J. R.; Wilson-mcneal, A.; Wright, S. L.; Galloway, T. S. Are We Underestimating Microplastic Abundance in the Marine Environment? A Comparison of Microplastic Capture with Nets of Different Mesh-Size. Environmental Pollution 2020, 265, 114721. https://doi.org/10.1016/j.envpol.2020.114721.

(22)	Leusch, F. Dl.; Lu, H.-C.; Perera, K.; Neale, P. A.; Ziajahromi, S. Analysis of the Literature Shows a Remarkably Consistent Relationship between Size and Abundance of Microplastics across Different Environmental Matrices. Environmental Pollution 2023, 319, 120984. https://doi.org/10.1016/j.envpol.2022.120984.

(23)	Wang, S.; Mintenig, S. M.; Wu, J.; Koelmans, A. A. Implications of Method- and Instrument-Based Size Detection Limits in μFTIR-Based Microplastic Analysis. Talanta 2026, 296, 128417. https://doi.org/10.1016/j.talanta.2025.128417.

(24)	Kooi, M.; Koelmans, A. A. Simplifying Microplastic via Continuous Probability Distributions for Size, Shape, and Density. Environ. Sci. Technol. Lett. 2019, 6 (9), 551–557. https://doi.org/10.1021/acs.estlett.9b00379.

(25)	Segur, T.; Hough, I.; Dobiasova, N.; Voisin, D.; Richon, C.; Angot, H.; Thomas, J. L.; Sonke, J. E. Using the Power Law Size Distribution to Extrapolate and Compare Microplastic Number and Mass Concentrations in Environmental Media. Micropl.&Nanopl. 2026, 6 (1), 45. https://doi.org/10.1186/s43591-026-00205-5.

(26)	Choy, C. A.; Robison, B. H.; Gagne, T. O.; Erwin, B.; Firl, E.; Halden, R. U.; Hamilton, J. A.; Katija, K.; Lisin, S. E.; Rolsky, C.; S. Van Houtan, K. The Vertical Distribution and Biological Transport of Marine Microplastics across the Epipelagic and Mesopelagic Water Column. Sci Rep 2019, 9 (1), 7843. https://doi.org/10.1038/s41598-019-44117-2.

(27)	Kashiwabara, L. M.; Kahane-Rapport, S. R.; King, C.; DeVogelaere, M.; Goldbogen, J. A.; Savoca, M. S. Microplastics and Microfibers in Surface Waters of Monterey Bay National Marine Sanctuary, California. Marine Pollution Bulletin 2021, 165, 112148. https://doi.org/10.1016/j.marpolbul.2021.112148.

(28)	Hamilton, B.; Rochman, C.; Hoellein, T.; Robison, B.; Van Houtan, K.; Choy, C. Prevalence of Microplastics and Anthropogenic Debris within a Deep-Sea Food Web. Marine Ecology Progress Series 2021, 675, 23–33. https://doi.org/10.3354/meps13846.

(29)	Michishita, S.; Gibble, C.; Tubbs, C.; Felton, R.; Gjeltema, J.; Lang, J.; Finkelstein, M. Microplastic in Northern Anchovies (Engraulis Mordax) and Common Murres (Uria Aalge) from the Monterey Bay, California USA - Insights into Prevalence, Composition, and Estrogenic Activity. Environmental Pollution 2023, 316, 120548. https://doi.org/10.1016/j.envpol.2022.120548.

(30)	Cowger, W.; Booth, A. M.; Hamilton, B. M.; Thaysen, C.; Primpke, S.; Munno, K.; Lusher, A. L.; Dehaut, A.; Vaz, V. P.; Liboiron, M.; Devriese, L. I.; Hermabessiere, L.; Rochman, C.; Athey, S. N.; Lynch, J. M.; De Frond, H.; Gray, A.; Jones, O. A. H.; Brander, S.; Steele, C.; Moore, S.; Sanchez, A.; Nel, H. Reporting Guidelines to Increase the Reproducibility and Comparability of Research on Microplastics. Applied Spectroscopy 2020, 74 (9), 1066–1077. https://doi.org/10.1177/0003702820930292.

(31)	Cowger, W.; Markley, L. A. T.; Moore, S.; Gray, A. B.; Upadhyay, K.; Koelmans, A. A. How Many Microplastics Do You Need to (Sub)Sample? Ecotoxicology and Environmental Safety 2024, 275, 116243. https://doi.org/10.1016/j.ecoenv.2024.116243.

(32)	Cole, M. A Novel Method for Preparing Microplastic Fibers. Sci Rep 2016, 6 (1), 34519. https://doi.org/10.1038/srep34519.

(33)	Hidalgo-Ruz, V.; Gutow, L.; Thompson, R. C.; Thiel, M. Microplastics in the Marine Environment: A Review of the Methods Used for Identification and Quantification. Environ. Sci. Technol. 2012, 46 (6), 3060–3075. https://doi.org/10.1021/es2031505.

(34)	Kooi, M.; Primpke, S.; Mintenig, S. M.; Lorenz, C.; Gerdts, G.; Koelmans, A. A. Characterizing the Multidimensionality of Microplastics across Environmental Compartments. Water Research 2021, 202 (July). https://doi.org/10.1016/j.watres.2021.117429.

(35)	DeCicco, L.; Hirsch, R.; Lorenz, D.; Read, J.; Walker, J.; Platt, L.; Watkins, D.; Blodgett, D.; Johnson, M.; Krall, A.; Stanish, L.; Zemmels, J.; Hinman, E.; Mahoney, M. dataRetrieval: Retrieval Functions for USGS and EPA Hydrology and Water Quality Data, 2025. https://cran.r-project.org/web/packages/dataRetrieval/index.html (accessed 2025-12-13).

(36)	Hoffman, M. J.; DiBenedetto, M. H. How Size-Dependent Settling Can Both Bias and Inform Microplastics Observations. Environ. Res. Commun. 2026, 8 (6), 065061. https://doi.org/10.1088/2515-7620/ae76df.

(37)	Cowger, W.; Karapetrova, A.; Lincoln, C.; Chamas, A.; Sherrod, H.; Leong, N.; Lasdin, K. S.; Knauss, C.; Teofilović, V.; Arienzo, M. M.; Steinmetz, Z.; Primpke, S.; Darjany, L.; Murphy-Hagan, C.; Moore, S.; Moore, C.; Lattin, G.; Gray, A.; Kozloski, R.; Bryksa, J.; Maurer, B. Open Specy 1.0: Automated (Hyper)Spectroscopy for Microplastics. Anal. Chem. 2025, 97 (32), 17345–17356. https://doi.org/10.1021/acs.analchem.5c00962.

(38)	Benjamini, Y.; Hochberg, Y. Controlling the False Discovery Rate: A Practical and Powerful Approach to Multiple Testing. Royal Statistical Society. Journal. Series B: Methodological 1995, 57 (1), 289–300. https://doi.org/10.1111/j.2517-6161.1995.tb02031.x.

(39)	Wickham, H.; Averick, M.; Bryan, J.; Chang, W.; McGowan, L. D.; François, R.; Grolemund, G.; Hayes, A.; Henry, L.; Hester, J.; Kuhn, M.; Pedersen, T. L.; Miller, E.; Bache, S. M.; Müller, K.; Ooms, J.; Robinson, D.; Seidel, D. P.; Spinu, V.; Takahashi, K.; Vaughan, D.; Wilke, C.; Woo, K.; Yutani, H. Welcome to the Tidyverse. Journal of Open Source Software 2019, 4 (43), 1686. https://doi.org/10.21105/joss.01686.

(40)	Oksanen, J.; Simpson, G. L.; Blanchet, F. G.; Kindt, R.; Legendre, P.; Minchin, P. R.; O’Hara, R. B.; Solymos, P.; Stevens, M. H. H.; Szoecs, E.; Wagner, H.; Barbour, M.; Bedward, M.; Bolker, B.; Borcard, D.; Borman, T.; Carvalho, G.; Chirico, M.; De Caceres, M.; Durand, S.; Evangelista, H. B. A.; FitzJohn, R.; Friendly, M.; Furneaux, B.; Hannigan, G.; Hill, M. O.; Lahti, L.; Martino, C.; McGlinn, D.; Ouellette, M.-H.; Ribeiro Cunha, E.; Smith, T.; Stier, A.; Ter Braak, C. J. F.; Weedon, J. Vegan: Community Ecology Package, 2001, 2.7-1. https://doi.org/10.32614/CRAN.package.vegan.

(41)	Fox, J.; Weisberg, S.; Price, B.; Adler, D.; Bates, D.; Baud-Bovy, G.; Bolker, B.; Ellison, S.; Firth, D.; Friendly, M.; Gorjanc, G.; Graves, S.; Heiberger, R.; Krivitsky, P.; Laboissiere, R.; Maechler, M.; Monette, G.; Murdoch, D.; Nilsson, H.; Ogle, D.; Proctor, I.; Ripley, B.; Short, T.; Venables, W.; Walker, S.; Winsemius, D.; Zeileis, A.; R-Core. Car: Companion to Applied Regression, 2026. https://cran.r-project.org/web/packages/car/index.html (accessed 2026-09-06).

(42)	Geyer, R.; Jambeck, J. R.; Law, K. L. Production, Use, and Fate of All Plastics Ever Made. Science Advances 2017, 3 (7), e1700782. https://doi.org/10.1126/sciadv.1700782.

(43)	Xia, F.; Wang, Y.; Wang, D.; Cai, Y.; Zhang, J. Seasonal Pulse Effect of Microplastics in the River Catchment-From Tributary Catchment to Mainstream. Journal of Environmental Management 2023, 342, 118316. https://doi.org/10.1016/j.jenvman.2023.118316.

(44)	Ross, M. S.; Loutan, A.; Groeneveld, T.; Molenaar, D.; Kroetch, K.; Bujaczek, T.; Kolter, S.; Moon, S.; Huynh, A.; Khayam, R.; Franczak, B. C.; Camm, E.; Arnold, V. I.; Ruecker, N. J. Estimated Discharge of Microplastics via Urban Stormwater during Individual Rain Events. Front. Environ. Sci. 2023, 11. https://doi.org/10.3389/fenvs.2023.1090267.

(45)	Werbowski, L. M.; Gilbreath, A. N.; Munno, K.; Zhu, X.; Grbic, J.; Wu, T.; Sutton, R.; Sedlak, M. D.; Deshpande, A. D.; Rochman, C. M. Urban Stormwater Runoff: A Major Pathway for Anthropogenic Particles, Black Rubbery Fragments, and Other Types of Microplastics to Urban Receiving Waters. ACS EST Water 2021, 1 (6), 1420–1428. https://doi.org/10.1021/acsestwater.1c00017.

(46)	Carr, S. A.; Liu, J.; Tesoro, A. G. Transport and Fate of Microplastic Particles in Wastewater Treatment Plants. Water Research 2016, 91, 174–182. https://doi.org/10.1016/j.watres.2016.01.002.

(47)	Mason, S. A.; Garneau, D.; Sutton, R.; Chu, Y.; Ehmann, K.; Barnes, J.; Fink, P.; Papazissimos, D.; Rogers, D. L. Microplastic Pollution Is Widely Detected in US Municipal Wastewater Treatment Plant Effluent. Environmental Pollution 2016, 218, 1045–1054. https://doi.org/10.1016/j.envpol.2016.08.056.

(48)	Mintenig, S. M.; Int-Veen, I.; Löder, M. G. J.; Primpke, S.; Gerdts, G. Identification of Microplastic in Effluents of Waste Water Treatment Plants Using Focal Plane Array-Based Micro-Fourier-Transform Infrared Imaging. Water Research 2017, 108, 365–372. https://doi.org/10.1016/j.watres.2016.11.015.

(49)	Botterell, Z. L. R.; Beaumont, N.; Dorrington, T.; Steinke, M.; Thompson, R. C.; Lindeque, P. K. Bioavailability and Effects of Microplastics on Marine Zooplankton: A Review. Environmental Pollution 2019, 245 (2019), 98–110. https://doi.org/10.1016/j.envpol.2018.10.065.

(50)	Savoca, M. S.; Murphy, E. L.; Brandon, A. M.; Thornton Hampton, L. M.; Roman, L.; Kong, M.; Fernandes, M. E.; Abrokwah, S.; Darter, C.; Galgani, F.; Provencher, J. F.; Alimba, C.; Santos, R. G.; Hardesty, B. D. The Effects of Plastic Pollution on Biodiversity. Nat. Rev. Biodivers. 2026, 2 (7), 425–440. https://doi.org/10.1038/s44358-026-00170-0.

Acknowledgements

We are deeply indebted to the volunteers that helped with both the labwork and fieldwork for this project.

Conflict of Interest Statement
The authors declare no conflict of interest

Code Availability

All data and code used in this study are available at https://github.com/mssavoca/W2W_river_plastics.